Original how-to by Afshin: http://forums.hybridz.org/topic/49087-z31300zx-ecumaf-to-280zxt-swap-guide-1981-supplement/

81T supplement by 280Z Turbo:  http://forums.hybridz.org/topic/26230-z31300zx-ecumaf-to-280zxt-swap-guide/

Wiring colors: b = black, g = green, l = blue, r = red, w = white, y = yellow

CONNECTING THE Z31 MAF:

1) Connect MAF Position A/wire 30 y/g to 30 y/b wire from 280ZXT harness
2) Connect MAF Position B/wire 31 b to 31 y/r wire from 280ZXT harness
3) Connect MAF Position C/wire black to body ground
4) Connect MAF Position D/wire 26 b to 26 b wire from 280ZXT harness
5) Connect MAF Position E/wire 27/35 b/w to any ignition activated + powered wire (pin 27 and 35 are + ignition wires on the same circuit, they are b/w (black with white strip) and can be tapped into from either b/w wire going to the 2 V.C.M solenoids that are located under the AFM bracket
6) Connect MAF Position F/wire 12 r/w (for cleaning cycle) to 33 y/l wire from 280ZXT harness

CONNECTING THE Z31 ECU:

1) Insert new pin into slot 12 of ECU connector then cut wire 33 y/l before it goes into ECU connector and attach to new pin #12. So 12 pin from the Z31 ECU thru wire 33 y/l will be connected to the 12 r/w wire on the MAF (marked as F). Now the ECU will be able to go into auto clean mode and keep the MAF filament clean.
2) Insert new pin into slot 34 of ECU connector then connect it to a new wire going to 12v switched power (27 / 35 black with white stripe).
3) If Z31 ignitor and solid state (SS) coil will be used, insert new pin into slot 3 of ECU connector then connect to a new wire going to the terminal of your coil. Otherwise this step can be skipped.

4) For 81T harness, insert new pin to #114, connect to wires from bypassed dropping resistor (direct battery power).

CONNECTING THE FUEL PUMP RELAY (FPR):

1) Cut black wire (- source used to turn on relay) right before green FPR, connect relay side of wire to new wire and take to pin 108 on the ECU connector. The Z31 ECU now controls the FPR ground.
2) Cut ECU pin 16 l/r ( + source for turning on relay) before the ECU connector end and connect to + ignition source (I tapped into the b/w wire going to my 280zxt fuel pump modulator which sits above the ECU).

CONNECTING THE AIR REGULATOR:

The air regulator was controlled thru pin 108 on the 280ZXT setup (which controls the FPR in the Z31 setup) as opposed to pin 16 on the Z31 setup. So I connected the 16 pin/wire coming out of the ECU connector to the 108 280ZXT harness wire (black which goes to the air regulator) and now the Z31 ECU will control the air regulator from ECU pin 16 thru the 108 wire. (While ECU pin 108 now controls the FPR).